DEMO Modes

LAUNDRY

**** REMOVE SHIPPING BOLTS IN BACK OF WASHER FIRST!! ****

WASHER FAST RPM DEMO (also is good for showing off TrueBalance™)

1) Press “Power/Spin/Soil” at same time,

2) Press “Start” (1st number is approx. 50 RPM, press play again and it will speed up to 600 RPM, press play a 3rd time and it speeds

up to 1150 RPM) (add a 0 to end of number shown on display).

SmartDiagnosis™ Demo

Washers – Power the unit, then press and hold “Wash/Rinse” button for 5 seconds

Dryers – Power the unit, then press and hold “Temp” button for 5 seconds

Wave Force™ Demo - works on all Top Loads with this motion

1) Press “Special Use/Soil Level/Power” and you will see T1 on the display.

2) Press “Start/Pause” repeatedly until you see T7 or T8

T7 = Motion - T8 = Shows RPM ramping to 1100 (add a 0 to end of number shown on display).

DRYER SteamFresh™ Demo for any unit that has this option

1) Fill Water Cartridge with tap water

2) Press “Power”, select “SteamFresh” and press “Start”

FRONT LOAD TurboWash™ Time Savings Demo - works on all models with this option

1) Turn the unit on, select “Cotton/Normal” or “Permanent Press” setting and make sure the “TurboWash” feature is also selected,

press play. The unit will then start to “Load Sense”. In just a minute it will give roughly a 30 minute wash time.

2) Pause the unit and deselect the “Turbo Wash” and the time increases to 52. Based on 10 loads a week, TurboWash will save the customer about 3 hours of wash time. This is a great easy way to show the benefits of TurboWash.

TOP LOAD Demo – WT5680 ONLY

Press “Power + Steam + TurboWash” – demonstrates spinning drum.

Tag On Feature Demo

- Download the Smart Diagnosis 2.0 app to your smart phone and utilize the NFC chip

- Power on the washer and hold your phone next to the control panel and go.

- Select what specific customized cycle to add to your wash cycles.

DEMO Modes

REFRIGERATION

Fresh Air Filter demo - You will see and hear the fan spinning

1) Locate door closure/light button (on right side of door by SmartDiagnosis™ speaker and on the left side in same location).

2) Depress both door closures/light buttons at the same time, it tricks the unit into thinking the door is closed and you will then see and hear the fan

spinning and drawing in air through the filter.

SmartDiagnosis™ demo

Open right side door - Hold “lock” button for three seconds then hold “freezer” for 5 seconds until panels goes blank and a 15 shows on the screen.

Ice maker demo

1) Open Ice maker door - Make sure the power to ice maker is on - Then take out ice bucket

2) On left hand side, directly under on/off button, up underneath the unit, in the very back, is a small white button, hold for 3 seconds till ice maker

starts moving.

Regular Refrigerator demo mode - How to turn the compressor off

• French Door w/dispenser – Press and hold down Ice Plus and Refrigerator buttons for 3 seconds with right side fridge door open.

• French Door without dispenser (LFC20770) – Press and hold down Ice plus button and arrow on left for 3 seconds till numbers start flashing 1,2,3 etc.

• French Door without dispenser – Press and hold down Ice Plus and Refrigerator buttons for 3 seconds with right side fridge door open.

• Side-by-Side – Press and hold down Ice Plus and Refrigerator buttons for 3 seconds with right side fridge door open.

• Bottom Mount (LBC24360 and LDC24370) - Press and hold down Ice Plus and Refrigerator buttons for 3 seconds.

• Top Mount – Set the temperature control to OFF.

• STUDIO - LSSB2791

How to turn the compressor off – Press and hold down Refrigerator and Freezer buttons for 3 seconds.

Smart Diagnosis – Press and hold down Freezer button for 3 seconds.

**** USE THE SAME FUNCTION TO TURN THE COMPRESSOR BACK ON ****